----------------------------------------------------------------------

Date: Tue, 18 Sep 90 15:20:25 PDT

From: Peter Denning <pjd@riacs.edu>

Subject: Arbitration Myths

In RISKS-10.3 of 13 Sept 1990 David Murphy says, "It is completely impossible

to build a fair arbiter or synchroniser."  He also says, "... any digital

design technique is just a way of improving engineering confidence in the

product, not of guaranteeing correctness."  These statements are not true.  I

hear them repeated frequently enough that I surmise they are part of the

folklore.  I write here with an antidote.

First, a definition: An arbiter is a device that selects exactly one out of a

set of requests represented as signals on input channels.  The arbiter must

work even if two or more of its inputs can change simultaneously.  This

behavior is considered "fair" since it does not give preference to any one

input.  Arbiters are used at the ports of memory modules, in the nodes of

interconnection networks, and in interrupt systems.  It is easy to build such

circuits.

The fundamental theorem of arbiters is: There is no fixed time bound for the

arbiter to make its choice.  The reason is that if two inputs change

simultaneously, the device can enter a metastable state from which it will exit

after a random amount of time (to be precise, the probability that the circuit

persists in the metastable state for more than t seconds is exp(-t/A), where A

is the mean switching time of the circuit).  If we as RISKS readers demand that

every arbitration be performed correctly, we must use asynchronous circuits

that will wait until an arbiter has settled.  These circuits are provably free

of synchronization errors.

But if we build an arbiter into a computer that assumes a fixed time for an

arbiter to reach every decision, there is a chance that the arbiter will not be

settled down and the half-signals at the arbiter's output may cause a

malfunction in the rest of the computer.  Although the probability of this

failure might seem small, failures every few days become very likely at the

clock speeds of modern computers.  If you are interested in reading more about

this, you can take a look at my American Scientist article (1985) and also

Chuck Seitz's chapter in the Mead and Conway book (1980).

Another common misunderstanding about arbitration concerns the lowest level at

which an indivisible operation must be implemented.  Dijkstra's solution to a

concurrent programming problem (1965) was a software arbiter that relied on the

existence of arbitration in the memory addressing circuits for individual

memory locations.  Lamport showed how to achieve fair mutual exclusion without

any requirement that references to memory cells are arbitrated (1974).

Conclusion: we know how to build fair arbiters and how to design

circuits that are free of synchronization errors.

READINGS

C. L. Seitz, Chapter 7, "System Timing," in Introduction to VLSI Systems, by C.

Mead and L. Conway, Addison-Wesley, 1980.

P. J. Denning, "The Arbitration Problem", American Scientist (Nov-Dec 1985).

L. Lamport, "A new solution of Dijkstra's concurrent programming problem",

Communications of ACM (August 197----------------------------------------------------------------------

Date: Mon, 24 Sep 90 10:36:11 -0700

From: lamport@src.dec.com (Leslie Lamport)

Subject: Arbitration Myths (Peter Denning, RISKS-10.42)

Peter Denning correctly observes:

The fundamental theorem of arbiters is: There is no fixed time bound

for the arbiter to make its choice.

It may make things a bit clearer to phrase this as follows.  A designer

of a circuit that has an arbiter has two choices:

1. Have his circuit wait arbitrarily long for the arbiter to make

up its mind, thereby guaranteeing that his digital circuit elements

will operate digitally, "seeing" only 0's and 1's.

2. Bound the length of time the circuit waits for the arbiter,

thereby introducing the possibility that his digital circuit

will behave like an analog circuit, with 1/2's flowing

along the wires, and do strange things.

Peter then asserts:

Lamport showed how to achieve fair mutual exclusion without any

requirement that references to memory cells are arbitrated (1974).

While perhaps correct, this statement is misleading.  The bakery algorithm

achieves mutual exclusion without assuming any arbitrated access to memory.

However, reading a memory cell that may be written while it is being read

requires that the reader have an arbiter.  (The reader needs an arbiter to

decide whether the change from 0 to 1 occurred before or after a read

operation.)  In fact, the mutual exclusion problem is characterized by the fact

that any solution requires an arbiter.  (It is a testimony to Dijkstra's

insight that certain requirements set down in his original 1965 paper,

apparently regarded as irrelevant and omitted by others in later statements of

the problem, are necessary to distinguish mutual exclusion from theoretically

easier problems that can be solved without an arbiter.)

As Peter observes, the only way to build a computer that is safe from

"arbitration failure" is by making choice 1, which means that the computer must

turn off its clock while waiting for an arbiter to decide.  Note that the

computer can't keep its clock ticking while waiting for the arbiter to make up

its mind, since it would then require another arbiter to decide within the

current clock cycle whether or not the first arbiter had made up its mind.  I

know of no computer that turns off its clock in this way.  Moreover, doing so

necessarily eliminates the possibility of fault-tolerance.  Omission faults can

be detected only by time-out, which means by keeping a clock running.  Thus, a

system that is impervious to arbitration failure cannot be fault-tolerant.

Thus, Peter's

Conclusion: we know how to build fair arbiters and how to design

circuits that are free of synchronization errors.

is again perhaps correct, but misleading.  We know how to design those

circuits, but we also know that they are impractical.

Having said all this, I should now add that the situation, although hopeless,

is not serious.  We cannot make circuits with no theoretical possibility of

arbitration failure, but we can make them with no practical possibility of such

failure.  By arguments that would constitute a proof to a physicist, and a joke

to a mathematician, I think one can show that if a circuit has normal reaction

time of order T0, then an optimimum arbiter has probability about e^(-T/T0) of

not having reached a decision by time T.  It appears possible to build such an

optimal arbiter.  Thus, by simply allowing enough time for the arbiter to

decide, the probability of an arbitration failure can be made negligibly small.

Of course, note the "can be".  How many of the engineers designing digitial

circuitry are aware of the problem?  Once, in the late 70's or early 80's, I

had the horrifying experience of spending 1/2 hour trying to explain the

problem to computer designers at Bendix, with an utter lack of success.

Leslie Lamport

P.S.  People interested in the arbiter problem might like to read my

unpublished paper "Buridan's Principle", available by request.

------------------------------

------------------------------

Date: Tue, 25 Sep 90 10:47:26 BST

From: Brian Randell <Brian.Randell@newcastle.ac.uk>

Subject: Arbiters

Nearly twenty years ago David Wheeler of Cambridge University, lectured here on

this subject in our Annual International Seminar on the Teaching of Computing

Science at University Level (7-10 Sept.  1971). RISKS readers might enjoy this

quote from the Seminar Report:

"The Problem of Synchronisation

Dr Wheeler devoted the rest of his talk to a discussion of a

particular problem in logical design. He chose to do this, rather than

give a more general talk, because he considers that discussion of this

point should form part of every course on hardware or logical design.

His reasons for emphasising this point, which he calls the problem of

synchronisation, are as follows:

(a) Many existing computers have faults because of neglect of this

point. (Dr Wheeler found that at least 50% of the computers whose

logical design he has studied in detail have faults of this kind.)

(b) The point is rarely taught well and only occasionally appears in

text books.

(c) It is apparently difficult to to appreciate. Furthermore, people

trained in switching theory or logical design find it especially

difficult.

(d) The problem is general. It is common to all forms of logic and may

also be present in systems programs. It touches many disciplines, for

example circuit theory, logical design, systems programming and

information theory.

(e) The occasional malfunctioning of all practical computers and

peripherals is to be expected if this point is neglected."

[The report then goes on to give a detailed account of David Wheeler's

lecture.]

(Younger RISKS readers may not be aware that David Wheeler, who I'm pleased to

say is still very active, was in 1949/50 the principal source of such concepts

as closed subroutines, assemblers, post mortems, and much else, in his

pioneering programming work on EDSAC, and went on to do much hardware design,

for example of EDSAC2 and of the Cambridge Ring.)

Brian Randell, Computing Laboratory, University of Newcastle upon Tyne, UK

PHONE = +44 91 222 7923    FAX = +44 91 222 8232 Brian.Randell@newcastle.ac.uk

------------------------------