Having seen an article (NYT Oct 12, 1988) on Nixdorf's "Voting Machine of theFuture", I made some inquiries which yielded the following.  The informationis probably not proprietary, but please don't redistribute it without mypermission.  The information below is based on casual conversation andcontains errors.There is a Federal Election Commission which promulgates standards for votingmachines.  NBS and MIL STD 810 are somehow involved.  There is a meeting soonwhere new voting booth regulations/criteria will be established.  Thecriteria will cover computerized versions, access by handicapped persons,....Nixdorf's system:PC/AT based.  No keyboard.  Stripped-down MSDOS built from Nixdorf source.Hardware version and software version go into escrow for a given election.Normal Software QA includes system build process.  THere is an "ITA"(Independent Test Auditor) who verifies the system, doing in excess of 7000ballots and photographing the tests.  Cabinet alarmed but alarm will notsound without power.  City or State normally apply mechnical seals to thecabinet.  There is a kInd of ignition key which has a 64 bit security code(password) unique to the city in which the machine is used. The key also hasa usage count (how many times the voting machine has been made ready).  Theclearing of memory is not up to NSA specs.  An individual timestamped ballotis stored in 3 locations plus a temporary location.  Checksums (the algorithmis not keyed) are used for integrity.  Cabinet door is locked and generates atimestamp audit record of some sort when opened.  Nixdorf seems to be almosttotally unaware of the TCSEC.Nixdorf's competition:Sequoia, "Shoop", BRC. All apparently use a tablet having pressure switcheswith a mylar overlay.  Simpler than a truly "computerized" system likeNixdorf's.The Nixdorf/New York City System (1992?):Voter Registration is already computerized.  There will be three stages onfuture election days.  The first is the Voting Machine described above.  TheVoting Machine results are impounded for 30 days, 90 if the election iscontested.  Upon poll closing the Voting Machine outputs a nonvolatile memorywhich is carried to a Tallying Station, also PC/AT based.  (In LA, theindividual Voting Machine results can be telecommunicated to the TallyingStation.)  The Tallying Station outputs a non-volatile memory which iscarried to a Back Office System which computes an Unofficial Official Result.Election Laws/Requirements:No Individual ballot may be tied back to the person who originated it.But it must be possible to recreate every ballot in case the results arechallenged.  Partial/Incremental results must not be inferrable.  Electionresults must be available within a couple of hours of poll closing.Contacts/Players:Penelope Bonsall FEC (202)-376-5670Ray Kuschell  Nixdorf Voting Machine Principal 617-273-0480Bob Nolte"George Brent"  of Georgetown (or Georgia Tech!):  Expert on Election Security.SRI International: consultant for recent new York City Voting Machine	acquisition.Bottom Line:I don't know.  Most of this was new to me.  Something here for MITRE?  CSD?Ray Kushell, source of most of the above, seemed willing to meet to share infoon our INFOSEC vis-a-vis his VoteSec.	-Fred